# S8 Soft Processor for Spartan-6 LX9

Complete Technical Specification and Implementation Plan (v1.1 — Farad-1 board verified)

S8 Project

October 2026

Table of Contents

# Document Control

| Field | Value |
| --- | --- |
| Title | S8 Soft Processor for Spartan-6 LX9 — Complete Technical Specification and Implementation Plan |
| Version | 1.1 (supersedes v1.0 and the original S8 Technical Implementation Manual) |
| Status | Implementation-ready specification. Design intent is frozen unless a change control entry says otherwise. |
| Target FPGA | XC6SLX9-2TQG144C on the Farad-1 board — verified (schematic renders + user manual + vendor UCF) |
| Target flash | W25Q64BV 64-Mbit SPI NOR — shared FPGA configuration flash (verified; S8 storage relocated, Section 13.4) |
| HDL | Verilog-2001, Xilinx ISE 14.7 flow |
| Host tools | Rust (stable), Cargo workspace |
| Audience | Any engineer or coding agent implementing the RTL, the host toolchain, or the verification environment from scratch |

## How to read this document

This specification is written so that an implementer with no prior context can build the complete S8 system — RTL, host toolchain, and verification — without consulting the original manual. The original manual is retained as background reading only; where the two documents disagree, this specification wins.

Reading order for a new implementer: Section 1 (scope) → Section 6 (ISA v1, frozen) → Section 7 (memory map) → Section 8 (CPU microarchitecture) → Section 9 (RTL modules) → Sections 10–13 (peripherals and boot) → Section 14 (Rust toolchain) → Section 16 (verification) → Section 18 (phase plan).

## Invariants (never violate these)

- The s8-isa Rust crate and the encoding tables in Section 6 are the single source of truth for instruction encodings. RTL localparams and assembler opcodes must both trace to those tables. If they ever disagree, one of them is wrong — fix it against Section 6, do not “average” the two.

- Every golden vector in Section 6.7 must be a regression test in both the Rust workspace and the RTL testbenches. The three starter-continuity words (0x1001, 0x5000, 0x4002) must pass in every environment.

- Never guess board facts. The Farad-1 fact sheet (Section 5) is verified against the schematic renders and is the only source of board truth; anything not listed there stays TODO until verified physically. The three remaining hardware-confirmation checks are listed in Section 5.3.

- All lengths, addresses, loop counts, and waits are bounded. Every FSM has a timeout; every host loop has a retry cap; every image is range-checked before use.

- Never erase or program the sector holding the FPGA configuration image if the flash turns out to be shared (Section 13.4).

- The active application slot is never erased while a replacement has not been fully committed. Commit order: erase → program payload → read back → program metadata marker last.

- CRC16 is integrity checking, not security. If adversarial uploads ever matter, add authentication; do not pretend CRC is a signature.

## Change log versus the original manual

The original manual was a sound plan with several internal inconsistencies. This version resolves all of them; the identifiers (P1–P6) refer to the analysis that found them.

| ID | Problem in original manual | Resolution in this document |
| --- | --- | --- |
| P1 | MMIO registers at 0xF0–0xF6 fell inside the proposed 1 KiB data RAM (0x000–0x3FF); the “decode outside RAM” rule was unsatisfiable. | Data address space is exactly 8 bits (256 B). RAM = 0x00–0xEF, MMIO = 0xF0–0xFF. Single decode rule: addr[7] & addr[6] & addr[5] (addr ≥ 0xF0) selects MMIO. The published 0xF0–0xF6 register map is preserved. |
| P2 | 16 KiB flash slots could never boot: program RAM is 8 KiB (4096 × 16-bit). | Slots are 8 KiB, exactly matching program RAM capacity. The assembler and the image packer both enforce the 8192-byte limit. |
| P3 | ISA v0 had no IN, no conditional branch, no ANDI, no peripheral-select field, and LDI could load only 0–63 — the manual’s own UART-echo example could not be assembled. | ISA v1 (Section 6) is frozen with IN, seven conditional branches, 8-bit LDI immediate, ALU-immediate group, peripheral port fields in IN/OUT, stack ops, and shifts. Every example program in this document assembles with v1 encodings, and the three useful starter vectors are preserved bit-for-bit. |
| P4 | CRC16 parameters and the ACK frame were unspecified. | CRC16-CCITT-FALSE is fully pinned (poly 0x1021, init 0xFFFF, no reflection, no final XOR, MSB-first) with the canonical check value 0x29B1 for ASCII “123456789”. The ACK frame layout and all eight status codes are byte-exact (Section 12). |
| P5 | The bootloader’s RUN command conflicted with the latched MODE semantics. | RUN performs a warm hand-off: CPU reset is released, the bootloader FSM parks in a terminal RUN state, and re-entering programming mode requires a hard reset with MODE=1 (Section 11.1). |
| P6 | GPIO width unstated; ISE 14.7 practicalities missing; several tables misaligned. | GPIO is exactly 4 bits, stated everywhere. Section 15 documents the ISE 14.7 environment (32-bit installer, VM guidance). All tables in this document were proofed in rendered form. |
| — | Metadata records for atomic update were both proposed in one 4 KiB sector, so erasing to update one record destroyed the other’s redundancy. | Two metadata records live in two separate 4 KiB sectors; a commit erases only the inactive record’s sector (Section 13.5). |
| B-facts | v1.0 left all board facts (B1–B9) open and assumed a logic-level USB-UART bridge and a 25 MHz default clock. | v1.1 verifies every fact against the Farad-1 schematic (page renders inspected visually): 50 MHz clock (p85), active-low debounced reset button (p87), MODE = DIP0 (p84, inverted), host link = FT2232H Channel B UART (p83/p82, one-time FT_PROG) with DB9/MAX232 fallback (p41/p40), and the W25Q64BV confirmed as the shared configuration flash — storage layout relocated to base 0x060000 (Section 13.4) and the full flash pin map pinned in the populated UCF (Section 15), including the no-pull-up WP#/HOLD# hazard (p62/p61 must be driven high). |

# 1. Introduction and scope

## 1.1 What S8 is

S8 is a small, custom 8-bit soft processor for learning and experimentation on a Spartan-6 LX9 FPGA. The FPGA holds the CPU, instruction and data storage, GPIO, UART, SPI flash controller, and boot control. A PC-side Rust toolchain assembles source code into an image and transfers it to the board over USB-UART. A W25Q64-family SPI NOR flash stores application images persistently so that new programs can be installed without rebuilding or re-transmitting the FPGA bitstream.

## 1.2 What this document guarantees, and what it does not

This document specifies the complete system at implementation level: encodings, memory maps, packet bytes, metadata records, module interfaces, FSM state lists, test inventories, and phase gates. Implementing it produces a working system without further design decisions, except the board facts listed in Section 5.

This document does not claim the system currently exists. As of v1.0, nothing beyond a Phase-0 learning skeleton (demo CPU with a hard-coded program, placeholder pins, a divided demo clock) has ever been synthesized. Sections 10–14 specify things that must be built and verified.

## 1.3 Non-goals (v1)

- No interrupts. All peripheral interaction is polling through status bits.

- No external memory bus. Program storage is internal BRAM only.

- No multiply/divide instructions, no barrel-shift-by-immediate encoding (shifts are by register value).

- No caches, no pipelines (the CPU is a multicycle FSM).

- No cryptographic authentication of uploads.

- No C-like compiler until Phase 7; the v1 toolchain is assembler-level only.

# 2. System architecture

| Layer | Responsibilities |
| --- | --- |
| PC / host | Rust CLI: assembler, image packer, serial uploader, disassembler, serial monitor |
| Host link | FT2232H: Channel A = USB JTAG (iMPACT); Channel B = USB-UART to S8 (p83/p82, after one-time FT_PROG); fallback DB9 RS-232 via MAX232 (p41/p40) |
| FPGA top | Clock/reset conditioning, synchronized MODE, CPU, memories, UART, GPIO, SPI, boot control |
| S8 CPU | Fetch → decode → execute; 8 × 8-bit registers; ALU; flags; 12-bit PC; 8-bit stack pointer |
| Peripherals | UART RX/TX with FIFOs, GPIO ports, SPI byte engine, W25Q64 command controller |
| W25Q64 | Nonvolatile metadata records, application slots A/B, image CRC |

Clocking policy. One primary clock domain for the entire design (clk = board oscillator). Baud generation, CPU slowdowns (if ever needed), and all timing use clock-enable pulses, never fabric-generated derived clocks. If a future requirement forces a second clock domain, explicit two-flop synchronizers and timing constraints are mandatory; no such requirement exists in v1.

Reset policy. One external active-low reset input rst_n, asserted asynchronously, deasserted synchronously through a two-flop reset synchronizer. Every sequential element uses the synchronized reset.

# 3. Project directory structure

s8-project/
  README.md
  docs/
    isa-v1.md                 # frozen copy of Section 6 (single source of truth)
    boot-protocol.md          # frozen copy of Section 12
    flash-layout.md           # frozen copy of Section 13.4-13.5
    board-bringup.md          # verified board facts as they are discovered
    verification-plan.md      # frozen copy of Section 16
  rtl/
    s8_top.v
    s8_cpu.v
    s8_alu.v
    s8_regfile.v
    s8_program_ram.v
    s8_data_ram.v
    s8_bus.v
    s8_gpio.v
    uart_rx.v
    uart_tx.v
    uart_regs.v
    spi_byte_engine.v
    w25q64_controller.v
    s8_boot_ctrl.v            # MODE handling + bootloader FSM + packet parser
    baud_gen.v
  tb/
    tb_s8_alu.v
    tb_s8_regfile.v
    tb_s8_cpu_programs.v
    tb_uart_loopback.v
    tb_spi_byte_engine.v
    tb_w25q64_model.v         # behavioral flash model used by other testbenches
    tb_boot_packets.v
    tb_full_boot.v
    vectors/                  # golden vectors exported from s8-isa (generated)
  constraints/
    board.ucf                 # populated only from verified schematic facts
  host/
    s8/                       # Rust workspace
      Cargo.toml
      crates/
        s8-isa/
        s8-assembler/
        s8-image/
        s8-serial/
        s8-disasm/
        s8-cli/
  examples/
    gpio.s8
    uart_echo.s8
    stack_demo.s8
    flash_info.s8
  tools/
    gen_isa_vectors.js        # emits golden vectors for both RTL and Rust tests
  build/                      # generated artifacts; bitstreams not committed by default

# 4. Top-level block diagram (textual)

+-------------------------------------------------------------+
   rst_n ------>| s8_top                                                      |
   clk   ------>|                                                             |
   mode  ------>|  [sync]---> boot_ctrl <----> uart_regs <----> uart_rx/tx <--|--> uart_rxd / uart_txd
   gpio[3:0] <->|    ^            |                ^                          |
                 |    |            v                |                          |
                 |  s8_cpu <--> s8_bus --> data_ram |                          |
                 |    |              |              |                          |
                 |    v              +--> gpio      |                          |
                 | program_ram                          |                          |
                 |    ^                                 |                          |
                 |    | (loader port)                  v                          |
                 |    +-------- w25q64_controller <--> spi_byte_engine -->| cs_n, sck, mosi, miso
                 +-------------------------------------------------------------+

Ownership rule: while MODE is latched as programming mode, the CPU is held in reset and the boot controller is the only master of program RAM, the SPI controller, and the UART data path. While running an application, the CPU owns program RAM, data RAM, and the UART/GPIO/SPI-register-visible peripherals; the flash controller is idle unless the application drives it through a future extension (not in v1: SPI registers are not CPU-visible in v1; flash access belongs to the boot controller only).

# 5. Board fact sheet — Farad-1 (VERIFIED in v1.1)

Target board: Farad-1 (Farad Andish, schematic rev 2016-12-29). In-repo evidence: BoardFiles/Farad-1-Schematic.pdf (6 sheets), BoardFiles/Farad-1-User-Manual.pdf, BoardFiles/Farad1_UCF.ucf. Every fact below was verified by inspecting page renders of the schematic (not text dumps — net-label/pin associations were confirmed visually), cross-checked against the vendor UCF and manual. This table is the single source of board truth for S8.

## 5.1 Verified facts (formerly open items B1–B9)

| # | Fact | Verified value | Evidence |
| --- | --- | --- | --- |
| B1 | FPGA part | XC6SLX9-2TQG144C → ISE part string xc6slx9-2tqg144 (speed grade −2, TQG144, commercial) | Schematic title blocks (all 6 sheets); user manual |
| B2 | System clock | 50.000 MHz oscillator (Y2) → p85 (net OSC, bank 1). PERIOD = 20.000 ns. External SMA clock on p88 exists but is unused. | Peripherals sheet (Y2); sheet 2 U1B; vendor UCF Clock = p85 |
| B3 | Reset | Button S2 → nFP_RST, p87, active low, 4.7 K pull-up (R22) + 4.7 µF (C46) — hardware-debounced. Maps directly to rst_n. S1 is PROGRAM_B; do not use it as a user reset. | Peripherals sheet; sheet 2 U1B (p87) |
| B4 | Host link | No logic-level USB-UART bridge exists. Two safe paths: (a) FT2232H Channel B as UART — BDBUS0 (TXD from FTDI) → net FT_D0 → p83 (FPGA in); BDBUS1 (RXD to FTDI) ← net FT_D1 ← p82 (FPGA out); 3.3 V CMOS; requires a one-time FT_PROG EEPROM change (Channel B = UART, no flow control; Channel A stays JTAG). (b) DB9 + MAX232 — FP_TX p41 → T1IN → DB9 pin 2 (straight-through cable); DB9 pin 3 → R1IN → R1OUT → divider R29 5.6 K / R30 10 K → p40 (≈3.2 V max — safe for the 3.3 V bank, input-only). Path (a) is the S8 default. | Peripherals sheet (MAX232 + divider inspected); sheet 2 U1C (FP_TX/FP_RX on p41/p40 — the vendor UCF’s Serial_Tx = p41, Serial_Rx = p40 is correct) |
| B5 | GPIO (v1) | LED0–LED3 = p105, p104, p102, p101, active HIGH (390 Ω → LED → GND). Extras (unconstrained in v1): 7-seg DISP0–7 = p100, 99, 98, 97, 95, 94, 93, 92 (active HIGH, common cathode); DIP1–3 = p81, p79, p74 (pull-ups, 0 = down); DS18B20 = p35; EXT_CLK = p88. | Peripherals sheet (LED/DISP/DIP polarity inspected); sheet 2 U1B/U1D; vendor UCF |
| B6 | MODE strap | No dedicated MODE pin. Use DIP0 = p84 (4.7 K pull-up, switch to GND: open = 1, closed = 0). Farad-1 adaptation: MODE_INVERT = 1 in s8_top, so switch open (all-off default) = run application, switch closed = programming. Spec semantics (logical 1 = programming) are unchanged. | Peripherals sheet (pull-ups); sheet 2 U1B (p84) |
| B7 | Flash sharing | SHARED: the W25Q64BV is the FPGA configuration flash, master SPI. MSEL straps: M0 tied to VCCO (1) at p69, M1 tied to GND (0) at p60, M2 unconnected (internal pull-down = 0). The bitstream occupies flash 0x000000 upward (~338 KB for the LX9). S8 storage is relocated (Section 13.4); the configuration region is untouchable. | User manual (.mcs flash-programming flow); sheet 2 U1C (straps inspected); sheet “System Memory” (U8) |
| B8 | Flash part and pins | W25Q64BV, JEDEC ID expected 0xEF 0x40 0x17. Wiring: SCK = p70 (CCLK, net FP_PR_CLK, board pull-up R23 4.7 K) → CK(6); CS# = p38 (CSO_B, net FP_CSO, board pull-up R6 4.7 K) → S(1); MOSI = p64 (net MISO0) → DI(5); MISO = p65 (net MISO1) ← DO(2); WP# = p62 (net MISO2) → pin 3 — no board pull-up: S8 must drive HIGH; HOLD# = p61 (net MISO3) → pin 7 — no board pull-up: S8 must drive HIGH. | System Memory sheet (U8 + pull-up placement inspected); sheet 2 U1C (pin↔net inspected) |
| B9 | I/O voltages | All four banks VCCO = +3.3 V (LM1117-3.3). Every S8 port: LVCMOS33. | FPGA Power + Power Regulator sheets |

## 5.2 Farad-1 pin table (S8-relevant)

| Function | Pin | Dir | Notes |
| --- | --- | --- | --- |
| clk | p85 | in | 50 MHz; PERIOD 20.000 ns |
| rst_n | p87 | in | Active low, RC-debounced button |
| mode | p84 | in | DIP0; inverted in RTL (open = run app) |
| gpio[3:0] | p105, p104, p102, p101 | out | LEDs, active high |
| uart_rxd | p83 | in | FTDI Ch.B TXD (path a) |
| uart_txd | p82 | out | FTDI Ch.B RXD (path a) |
| uart2_rxd | p40 | in | DB9 via MAX232 + 5K6/10K divider (path b) |
| uart2_txd | p41 | out | DB9 via MAX232 |
| spi_sck | p70 | out | Flash CK (CCLK pin, net FP_PR_CLK) |
| spi_cs_n | p38 | out | Flash CS# (CSO_B; board pull-up R6) |
| spi_mosi | p64 | out | Flash DI (net MISO0) |
| spi_miso | p65 | in | Flash DO (net MISO1) |
| spi_wp_n | p62 | out | Drive HIGH — no board pull-up |
| spi_hold_n | p61 | out | Drive HIGH — no board pull-up |

Left unconstrained and unused in v1: SRAM IS61WV5128 (512 KiB × 8: MEM_D0-7 = p142, 141, 140, 139, 112, 111, 119, 120; ADD0-18 = p6, 5, 2, 1, 144, 137, 134, 133, 132, 131, 118, 117, 116, 115, 114, 123, 124, 126, 127; MEM_CE = p143, MEM_WE = p138, MEM_OE = p121 — reserved for a future XIP/data-expansion phase), 7-segment, DIP1–3, DS18B20, EXT_CLK, FTDI FIFO handshake lines.

## 5.3 Remaining hardware checks (bring-up time; not design unknowns)

- Flash-pin confirmation: the B8 map was read from schematic renders; confirm it on hardware with the Phase-5 JEDEC read. If 0x9F fails, swap p64/p65 (MOSI/MISO) first; a power-off continuity check (flash pin ↔ FPGA pin, diode mode) settles it in minutes.

- FT_PROG step: back up the FT2232H EEPROM, then set Channel B function = UART (no flow control) once. Record the backup in docs/board-bringup.md.

- VCCAUX: vendor power design (2.5 V per UG380 expectations); S8 does not use it — recorded so nobody wires analog or aux-dependent logic into the plan.

## 5.4 Spartan-6 LX9 resource budget (XC6SLX9-2TQG144C, DS160 family table)

| Resource | Device total | S8 v1 use | Remaining |
| --- | --- | --- | --- |
| Logic cells | 9,152 | est. 3,000–4,500 (CPU + UART + SPI + boot control) | ample |
| Slices (4 LUT + 8 FF each) | 1,430 | est. 700–900 ≈ 50–60 % (budget gate: ≤ 60 %, Section 16.5) | ~500+ for compiler/debugger phases |
| Block RAM (18 Kb blocks) | 32 (576 Kb total) | 4 — program RAM (4096 × 16 = 64 Kb in 2048×9 mode, 2 wide × 2 deep) | 28 |
| Distributed RAM (max) | 90 Kb | small — data RAM (240 × 8) + 2 × 16-byte UART FIFOs (~32 LUTs) | ample |
| DSP48A1 slices | 16 | 0 (no multiply instruction in ISA v1) | all 16 |
| Global clock nets used | 4 × 16 (4 clock regions) | 1 (oscillator, p85); no CMT/DCM — baud via phase accumulator | — |
| CMTs (DCM + PLL) | 2 | 0 | — |
| User I/O (TQG144 package) | 102 | ~12 S8 ports (clk, rst, mode, 4 LED, 2 UART, 4–6 SPI) + SRAM/7-seg/DIP reserved | rest unused in v1 |

Notes: the design fits comfortably; the tightest axis is slices (not BRAM). If utilization approaches the 60 % gate, the first reductions are: RX FIFO depth 16 → 8, optional GPIO_DIR register removal, and the ROL/ROR shift variants (funct space already reserved). BRAM usage is deliberately conservative so a future XIP/instruction-fetch phase (Section 19.2) can claim 4–8 more blocks. Sources: DS160 family overview table (logic cells / slices / FFs / distributed RAM / DSP / BRAM per device) — cross-checked against two independent part summaries.

# 6. Instruction set architecture v1 (FROZEN)

Version note: v1 replaces the unfrozen starter encoding. Three starter instructions keep their exact words (LDI R0,1 = 0x1001, OUT GPIO_OUT,R0 = 0x5000, JMP 0x002 = 0x4002) so starter testbenches still carry value. Any future change requires ISA v2, a new document revision, and regeneration of all golden vectors.

## 6.1 Programmer’s model

| Resource | Size | Notes |
| --- | --- | --- |
| R0–R7 | 8 × 8-bit | General purpose. R0 is not hardwired to zero. No register is reserved by hardware; the ABI (Section 19.3) reserves some by convention. |
| PC | 12-bit | Word address, range 0x000–0xFFF. Instruction words are 16 bits. |
| SP | 8-bit | Stack pointer, byte address in data space. Reset value 0xF0 (empty stack). |
| Flags | Z, C, N, O | Zero, Carry, Negative (bit 7), oVerflow (signed). Updated only by instructions listed in 6.5. |
| Halt bit | 1 | Set by HALT; cleared by reset or by writing CPU_CONTROL.resume. |
| Trap bit | 1 | Set by illegal opcode / illegal branch condition / stack fault; cleared only by reset. |

## 6.2 Instruction formats

All instructions are exactly 16 bits. op = bits [15:12].

| Format | Fields | Used by |
| --- | --- | --- |
| R (register) | op[15:12], rd[11:9], rs[8:6], funct[5:3], 000[2:0] | ALU reg-reg, SHIFT |
| I8 (immediate 8) | op[15:12], rd[11:9], imm8[7:0] | LDI |
| RI (reg + imm6) | op[15:12], rd[11:9], funct[8:7], 0[6], imm6[5:0] | ADDI/ANDI/ORI/CMPI |
| M (memory) | op[15:12], rd[11:9], rs[8:6] | LD, ST |
| A (address) | op[15:12], addr12[11:0] | JMP, CALL |
| P (port) | OUT: op, port[11:9], rs[8:6]. IN: op, rd[11:9], port[8:6] | IN, OUT |
| B (branch) | op[15:12], cond[11:9], rel8[7:0] | Jcc |
| S (single) | op[15:12], 000000000000 | RET, HALT |
| U (unary reg) | op[15:12], rd[11:9], 000000000 | PUSH, POP |

Unused bits must be encoded as zero. Assemblers must emit zero for them; decoders must treat non-zero unused bits as an illegal opcode (trap), so that encoding drift is detected early rather than silently tolerated.

## 6.3 Opcode map

| op | Mnemonic | Format | Semantics summary |
| --- | --- | --- | --- |
| 0x0 | ALU reg-reg | R | ADD, SUB, AND, OR, XOR, CMP, MOV, NOT (funct 0–7) |
| 0x1 | LDI | I8 | rd ← zero-extend(imm8) |
| 0x2 | ALU imm | RI | ADDI, ANDI, ORI, CMPI (funct 0–3), imm6 |
| 0x3 | LD | M | rd ← M[rs] |
| 0x4 | JMP | A | PC ← addr12 |
| 0x5 | OUT | P | Port[port] ← Rs |
| 0x6 | Jcc | B | Conditional PC ← PC + 1 + sext(rel8) |
| 0x7 | CALL | A | Push return address; PC ← addr12 |
| 0x8 | ST | M | M[rs] ← rd |
| 0x9 | RET | S | PC ← pop |
| 0xA | IN | P | rd ← Port[port] |
| 0xB | PUSH | U | Push rd |
| 0xC | POP | U | Pop into rd |
| 0xD | SHIFT | R | SHL, SHR, ROL, ROR (funct 0–3), amount = Rs[2:0] |
| 0xE | reserved | — | Illegal opcode → TRAP |
| 0xF | HALT | S | Halt until reset or resume |

## 6.4 Instruction reference

Field names below refer to Section 6.2. M[x] is the data-space byte at address x. sext is sign extension to 8 bits. PC_after = address of the instruction following the current one (PC + 1, wrapping within 12 bits).

| Mnemonic | Encoding | Semantics | Flags |
| --- | --- | --- | --- |
| ADD rd, rs | 0x0, funct=0 | {C, rd} ← rd + rs (9-bit sum) | Z C N O |
| SUB rd, rs | 0x0, funct=1 | {B, rd} ← rd − rs; C = NOT borrow (C=1 iff rd ≥ rs) | Z C N O |
| AND rd, rs | 0x0, funct=2 | rd ← rd & rs | Z, N; C←0, O←0 |
| OR rd, rs | 0x0, funct=3 | rd ← rd ¦ rs | Z, N; C←0, O←0 |
| XOR rd, rs | 0x0, funct=4 | rd ← rd ^ rs | Z, N; C←0, O←0 |
| CMP rd, rs | 0x0, funct=5 | Compute rd − rs; discard result, keep flags only | Z C N O |
| MOV rd, rs | 0x0, funct=6 | rd ← rs | Z, N; C←0, O←0 |
| NOT rd, rs | 0x0, funct=7 | rd ← ~rs (note: source is rs, not rd) | Z, N; C←0, O←0 |
| LDI rd, imm8 | 0x1 | rd ← imm8 | Z, N (of result); C←0, O←0 |
| ADDI rd, imm6 | 0x2, funct=0 | rd ← rd + zext(imm6) | Z C N O |
| ANDI rd, imm6 | 0x2, funct=1 | rd ← rd & zext(imm6) | Z, N; C←0, O←0 |
| ORI rd, imm6 | 0x2, funct=2 | rd ← rd ¦ zext(imm6) | Z, N; C←0, O←0 |
| CMPI rd, imm6 | 0x2, funct=3 | flags only, from rd − zext(imm6) | Z C N O |
| LD rd, [rs] | 0x3 | rd ← M[rs] | unchanged |
| ST rd, [rs] | 0x8 | M[rs] ← rd (base in rs, data in rd — mirrors LD) | unchanged |
| JMP addr12 | 0x4 | PC ← addr12 | unchanged |
| JZ / JNZ / JC / JNC / JN / JP / JO rel8 | 0x6, cond 0–6 | if condition: PC ← PC_after + sext(rel8) | unchanged |
| (cond=7) | 0x6 | Illegal → TRAP | unchanged |
| CALL addr12 | 0x7 | push PC_after; PC ← addr12 | unchanged |
| RET | 0x9 | PC ← pop | unchanged |
| IN rd, port | 0xA | rd ← Port[port] | unchanged |
| OUT port, rs | 0x5 | Port[port] ← rs | unchanged |
| PUSH rd | 0xB | push rd | unchanged |
| POP rd | 0xC | rd ← pop | unchanged |
| SHL rd, rs | 0xD, funct=0 | {C, rd} ← rd << Rs[2:0] (C = last bit out) | Z N C; O←0 |
| SHR rd, rs | 0xD, funct=1 | logical right shift; C = last bit out | Z N C; O←0 |
| ROL rd, rs | 0xD, funct=2 | rotate left through rd, no C participation | Z N; C←0, O←0 |
| ROR rd, rs | 0xD, funct=3 | rotate right through rd, no C participation | Z N; C←0, O←0 |
| HALT | 0xF | Halt bit ← 1; CPU stops retiring until resume or reset | unchanged |

Semantics notes.

- Branch targets: target = (PC_after + sext(rel8)) mod 4096. The assembler computes rel8 = target_label − PC_after. Range −128..+127 from PC_after; out-of-range is an assembler error. Automatic relaxation (inverted branch over a JMP) is specified as assembler behavior for v1: when a Jcc target is out of range, the assembler emits inverted-cc +3; JMP target and reports the rewrite in verbose mode.

- LD/ST addresses are full 8-bit data-space addresses held in rs; the MMIO region 0xF0–0xFF is reachable by ordinary LD/ST but the canonical access path is IN/OUT (identical hardware decode; IN/OUT exist for code density and clarity).

- Shift amounts use only rs[2:0]; a shift by 8 behaves as a shift by 0 (copies rd to rd, sets Z/N from the unchanged value, C←0 for SHL/SHR).

## 6.5 Flag formulas (normative)

Let a, b be the 8-bit operands, and r the 8-bit result.

- ADD/ADDI: sum9 = {1'b0,a} + {1'b0,b}; r = sum9[7:0]; C = sum9[8]; Z = (r == 0); N = r[7]; O = (a[7] == b[7]) && (r[7] != a[7]).

- SUB/CMP/CMPI: diff9 = {1'b0,a} - {1'b0,b}; r = diff9[7:0]; C = ~diff9[8] (C = 1 when a ≥ b, unsigned); Z, N from r; O = (a[7] != b[7]) && (r[7] != a[7]).

- AND/OR/XOR/NOT/MOV/LDI: Z, N from result; C = 0; O = 0.

- SHL: C = last bit shifted out (the bit leaving position 7); SHR likewise from position 0; ROL/ROR: C = 0.

- All other instructions leave every flag unchanged.

- Reset clears Z, C, N, O.

## 6.6 HALT, TRAP, and stack faults

- HALT sets the halt bit. The CPU remains in the HALT state, keeps servicing nothing, and exits only via CPU_CONTROL.resume (port 6, bit 0) or reset. Halt is a legitimate program state: BOOT_STATUS and CPU_CONTROL remain readable while halted.

- TRAP is entered on: opcode 0xE; any R-format instruction with non-zero bits [2:0]; RI-format with bit 6 set; any unused field pattern not defined above; branch condition 7; PUSH/POP/CALL/RET when SP is outside the legal range 0xE0–0xF0 before or after the operation. TRAP latches the trap bit; the CPU stops retiring until reset. Trap is not resumable by design: it indicates a bug, and the debug strategy is reset + BOOT_STATUS/CPU_CONTROL inspection.

- Stack bounds: SP starts at 0xF0 (empty). PUSH/CALL decrement before use; POP/RET read then increment. Usable depth is 16 entries (0xEF down to 0xE0). A faulting stack operation leaves SP and memory unmodified (check before commit).

## 6.7 Golden encoding vectors

These words are normative. They were machine-generated from the encoding rules above and must appear verbatim as pass criteria in the Rust tests (s8-isa) and the RTL instruction-vector testbench.

| Assembly | Word |
| --- | --- |
| LDI R0, 1 | 0x1001 |
| OUT GPIO_OUT, R0 | 0x5000 |
| JMP 0x002 | 0x4002 |
| ADD R2, R3 | 0x04C0 |
| SUB R7, R1 | 0x0E48 |
| AND R4, R5 | 0x0950 |
| OR R0, R7 | 0x01D8 |
| XOR R6, R6 | 0x0DA0 |
| CMP R1, R2 | 0x02A8 |
| MOV R1, R0 | 0x0230 |
| NOT R4 | 0x0838 |
| NOP (= MOV R0, R0) | 0x0030 |
| LDI R7, 255 | 0x1EFF |
| LDI R3, 0xA5 | 0x16A5 |
| ADDI R0, 5 | 0x2005 |
| ANDI R1, 0x0F | 0x228F |
| ORI R2, 0x30 | 0x2530 |
| CMPI R1, 0 | 0x2380 |
| LD R1, [R0] | 0x3200 |
| LD R5, [R7] | 0x3BC0 |
| ST R1, [R0] | 0x8200 |
| ST R0, [R7] | 0x81C0 |
| JMP 0xFFF | 0x4FFF |
| CALL 0x123 | 0x7123 |
| RET | 0x9000 |
| JZ .-3 | 0x60FD |
| JNZ .-1 | 0x62FF |
| JC .+127 | 0x647F |
| JN .-128 | 0x6880 |
| IN R1, UART_STATUS | 0xA2C0 |
| IN R0, UART_DATA | 0xA100 |
| IN R2, GPIO_IN | 0xA440 |
| OUT UART_DATA, R0 | 0x5800 |
| OUT GPIO_DIR, R3 | 0x54C0 |
| PUSH R5 | 0xBA00 |
| POP R5 | 0xCA00 |
| SHL R2, R3 | 0xD4C0 |
| SHR R2, R3 | 0xD4C8 |
| ROL R1, R4 | 0xD310 |
| ROR R1, R4 | 0xD318 |
| HALT | 0xF000 |

Note: instruction word 0x0000 is ADD R0, R0, a real instruction, not a NOP. Programs must never rely on zero-filled memory executing as NOP. The assembler pseudo-op NOP emits 0x0030 (MOV R0, R0).

The generator that produced this table (tools/gen_isa_vectors.js) also exports machine-readable vectors to tb/vectors/ for the RTL testbenches and to the Rust test data; regenerate after any approved ISA change.

## 6.8 Assembler syntax

- Case-insensitive mnemonics and register names (R0–R7).

- Comments start with ; and run to end of line.

- Labels: name: at the start of a line, bound to the next emitted word address.

- Numbers: decimal (42), hex (0x2A), binary (0b101010). Negative decimals allowed for relative branch operands.

- Register indirect operands: [Rn].

- Branch operands: label or explicit relative value (.-3, .+2, or a raw number interpreted as relative offset — labels are strongly preferred).

- Port names are predefined symbols: GPIO_OUT=0, GPIO_IN=1, GPIO_DIR=2, UART_STATUS=3, UART_DATA=4, BOOT_STATUS=5, CPU_CONTROL=6.

- Directives: .org <word_addr> (set location counter for program memory), .word <w>[, <w>...] (emit raw 16-bit words), .byte <b>[, <b>...] (emit bytes, packed two per word, high byte first), .equ <name>, <value> (constant), .ascii "text" (emit bytes, high-byte-first packing, same rule as .byte).

- Pseudo-ops expanded by the assembler: NOP → MOV R0,R0; BRA label → JMP label; LDI16 rd, <16-bit> → LDI rd, hi8; ORI rd, lo8 — rejected unless rd is documented as scratch by the programmer (it overwrites rd twice; emitted with a warning).

- Errors are mandatory (with file, line, column, and expected-vs-found): unknown mnemonic, bad register, immediate out of range for its format (imm8 > 255, imm6 > 63), duplicate label, undefined label, branch target out of relative range after relaxation, .org backwards overlap, image exceeding 8192 bytes.

## 6.9 Worked programs (normative listings)

### 6.9.1 GPIO blink-seed (3 words)

LDI  R0, 1          ; 0x1001
        OUT  GPIO_OUT, R0   ; 0x5000
loop:   JMP  loop           ; 0x4002

Words: 0x1001 0x5000 0x4002. This is the hardware smoke test for Phase 2.

### 6.9.2 UART echo (9 words)

start:  IN   R1, UART_STATUS  ; 0xA2C0
        ANDI R1, 0x01         ; 0x2281   ; RX_VALID
        JZ   start            ; 0x60FD
        IN   R0, UART_DATA    ; 0xA100   ; pop RX FIFO
wait:   IN   R1, UART_STATUS  ; 0xA2C0
        ANDI R1, 0x02         ; 0x2282   ; TX_READY
        JZ   wait             ; 0x60FD
        OUT  UART_DATA, R0    ; 0x5800
        JMP  start            ; 0x4000

Words: 0xA2C0 0x2281 0x60FD 0xA100 0xA2C0 0x2282 0x60FD 0x5800 0x4000. Both relative branches resolve to −3.

### 6.9.3 Stack / CALL-RET demo

LDI  R1, 5
        CALL work              ; R1 = 10 after return
        HALT
work:   PUSH R1
        ADD  R1, R1
        POP  R1
        ; ... recompute: the demo doubles R1 then restores — shown to exercise
        ; CALL/RET/PUSH/POP encodings; real code uses the ABI in Section 19.3.
        RET

The assembler test suite must round-trip this program and check it against hand-computed words.

# 7. Memory architecture

## 7.1 Program memory

- 4096 × 16-bit synchronous BRAM (8 KiB), word-addressed by the 12-bit PC.

- Single read port shared between the CPU fetch path and the boot loader. The loader has priority only while the CPU is held in reset; the mux in s8_program_ram must never grant both in the same cycle (assertion in simulation).

- Synchronous read: address registered at the clock edge, data valid on the following edge. The CPU FSM accounts for this latency (Section 8.3).

- Byte order within a word: the byte at even image offset is the high byte (bits [15:8]). A word at address k holds image bytes 2k (high) and 2k+1 (low). All of assembler output, .s8img payloads, flash bytes, and loader logic obey this single rule.

- No write port is exposed to the application in v1; only the loader writes program RAM.

## 7.2 Data memory

- Data address space is 8 bits (256 bytes total).

- s8_data_ram: 240 × 8 synchronous RAM at 0x00–0xEF. Same single-cycle sync-read semantics as program RAM.

- Software convention (not enforced by hardware beyond stack bounds): 0x00–0xDF variables, 0xE0–0xEF stack region.

- 0xF0–0xFF is MMIO, decoded by s8_bus outside the RAM chip select. 0xF8–0xFF are reserved: reads return 0x00, writes are ignored, no trap.

## 7.3 MMIO register map (ports 0–7 at addresses 0xF0–0xF7)

| Addr | Port | Name | Access | Definition |
| --- | --- | --- | --- | --- |
| 0xF0 | 0 | GPIO_OUT | R/W | Bits [3:0] drive GPIO pads when the corresponding DIR bit is 1. Reset value 0x00. |
| 0xF1 | 1 | GPIO_IN | R | Bits [3:0]: two-flop-synchronized pad samples. Read-only. |
| 0xF2 | 2 | GPIO_DIR | R/W | Bit = 1 → pad is an output. Reset value 0x0F (all outputs; keeps the LED demo working out of reset). |
| 0xF3 | 3 | UART_STATUS | R | bit0 RX_VALID (RX FIFO non-empty); bit1 TX_READY (TX FIFO not full); bit2 RX_FRAME_ERR (sticky, cleared by reading UART_DATA); bit3 RX_OVERRUN (sticky, cleared by reading UART_DATA). |
| 0xF4 | 4 | UART_DATA | R/W | Read: pop one byte from RX FIFO (returns 0x00 if empty; clears sticky error bits). Write: push one byte to TX FIFO (write ignored when TX FIFO full). |
| 0xF5 | 5 | BOOT_STATUS | R | bit0 MODE_LATCHED (1 = programming mode); bit1 IMAGE_VALID (a valid image was found at boot); bits[4:2] BOOT_ERR: 0 none, 1 no valid image, 2 image CRC mismatch, 3 flash ID mismatch, 4 flash timeout. |
| 0xF6 | 6 | CPU_CONTROL | R/W | Write: bit0 = 1 → clear halt (resume); bit1 = 1 → soft reset request (re-enters the boot flow with latched MODE). Read: bit0 HALTED, bit1 TRAPPED, bit2 RUNNING. |
| 0xF7 | 7 | — | — | Reserved, reads 0x00. |

Reads of write-only fields and writes to read-only fields are benign (return 0 / ignored). All GPIO inputs are synchronized; no combinational path from any pad reaches CPU control logic.

# 8. CPU microarchitecture

## 8.1 Datapath

- 8 × 8-bit register file, two read ports, one write port, synchronous write, asynchronous read (distributed RAM; the file is small).

- ALU: combinational; computes result and the four flags per Section 6.5. Flags live in a flag register updated at the EXECUTE→next transition when the decoded instruction writes flags.

- PC: 12-bit, with increment, jump, branch, call (push PC_after), and return (pop) paths.

- SP: 8-bit with increment/decrement and bounds comparator against 0xE0–0xF0.

- Imm extraction unit: imm8, imm6 (zero-extended), addr12, rel8 (sign-extended), all from the latched instruction word.

## 8.2 Control FSM (multicycle, one instruction per state sequence)

| State | Action | Next |
| --- | --- | --- |
| RESET | Outputs held safe; PC ← 0; SP ← 0xF0; flags cleared | FETCH_REQ |
| FETCH_REQ | Drive program-RAM address = PC | FETCH_WAIT |
| FETCH_WAIT | Wait one cycle for synchronous read; latch instruction word | DECODE |
| DECODE | Decode opcode/funct; validate unused bits; extract immediates | EXECUTE, or TRAP on illegal encoding |
| EXECUTE | ALU operation; register file write; flag update; or port read via IN | MEM (for LD/ST/PUSH/POP/CALL/RET stack access), otherwise FETCH_REQ |
| MEM | Drive data bus: address, we/re; one cycle for synchronous RAM or MMIO | FETCH_REQ (writeback folded into MEM) |
| HALT | CPU idle; BOOT_STATUS/CPU_CONTROL readable | FETCH_REQ on resume, RESET on soft reset |
| TRAP | Latched stop | RESET only |

Any state combined with an active reset returns to RESET. The multicycle structure means one instruction takes 3–5 cycles; there is no requirement to optimize this in v1. Correctness first: every state must be able to reach HALT or TRAP when the opcode demands it, and no state may retire two instructions.

## 8.3 Bus interface and wait states

The CPU presents to s8_bus: addr[7:0], re, we, wdata[7:0]; the bus returns rdata[7:0] and ready. RAM targets answer ready in one cycle (the MEM state already waits one cycle). MMIO targets answer combinationally in the same cycle. If a future peripheral needs a wait state, it holds ready low and the CPU stays in MEM; v1 has no such peripheral, but the FSM must implement the wait so the interface is future-proof. Writes to RAM/MMIO complete only when ready is seen.

## 8.4 Clock and reset

Single clock clk (Farad-1: 50.000 MHz oscillator on p85 — CLK_HZ = 50000000 on hardware, any value in simulation). Reset: asynchronous assert, synchronous deassert, two-flop synchronizer, active low (p87 button is active low with hardware debounce). The demo divider from the starter design is removed; the CPU runs at full clock with no slow-down in v1.

# 9. RTL module specifications

Common conventions: Verilog-2001; every module has a synchronous reset unless purely combinational; parameters for anything board- or capacity-related; one module = one file; no latches (assert in synthesis review); registered outputs where a path would otherwise be long.

| Module | Key ports | Specification |
| --- | --- | --- |
| s8_top | clk, rst_n, mode, uart_rxd, uart_txd, gpio[3:0], cs_n, sck, mosi, miso | Wiring only. Instantiates and connects everything; owns the MODE synchronizer and the boot-mode latch. Zero guessed pin numbers (UCF only). |
| s8_cpu | instr[15:0], instr_addr[11:0], bus interface, halt/trap status | FSM per Section 8; exposes halted, trapped for CPU_CONTROL. |
| s8_alu | op_a, op_b[7:0], funct, carry-in context, result, flags_out | Combinational; implements Section 6.5 formulas exactly; unit-tested exhaustively per operation class against a software model. |
| s8_regfile | raddr_a, raddr_b, waddr, wdata, we, clk | 8 × 8; sync write; async read; simulation-friendly: expose registers to TB via hierarchical reference, not extra ports. |
| s8_program_ram | cpu_addr, cpu_out (unused in v1), load_addr, load_data, load_we, sel_load | 4096 × 16 sync BRAM; read-first; loader port muxed with CPU port; assertion: never both enables in one cycle. |
| s8_data_ram | addr[7:0], re, we, wdata, rdata | 240 × 8 sync RAM, decode excludes 0xF0+ (chip-select input from bus). |
| s8_bus | cpu bus in; RAM chips selects; MMIO read mux | Address decode per Section 7.2; read mux prioritizes ready sources; never routes 0xF0+ to RAM. |
| s8_gpio | gpio_out, gpio_dir, pad_in, pad_out, pad_oe | 4-bit; synchronizes pad_in (2FF); drives pads per DIR; safe default (DIR=0x0F, OUT=0x00 at reset). |
| baud_gen | clk, tick16 (16× baud tick), parameter CLK_HZ, BAUD | Phase accumulator (Section 10.1); fully parameterized. |
| uart_rx | rxd (already synchronized at top), tick16, rdata, valid, framing_err, overrun | Start-bit qualification (half-bit sample must still be low), 8 data LSB-first, 1 stop bit; framing error flag on bad stop; overrun when pushing a full FIFO (flag latched in uart_regs). |
| uart_tx | tdata, tvalid, tready, tick16, txd | 8N1, LSB-first; ready/valid handshake; deterministic one-byte-per-10-bit-times throughput. |
| uart_regs | cpu bus side, rx/tx FIFO sides, status | 16-byte RX FIFO, 16-byte TX FIFO (BRAM or distributed per utilization review); sticky error bits cleared on UART_DATA read; implements port 3 and 4 semantics exactly. |
| spi_byte_engine | start, tx_byte, rx_byte, done, sck, mosi, miso, cpol/cpha fixed 0/0 | Shifts 8 bits MSB-first; SCLK = clk divided by 2×DIV (DIV default 4 → 6.25 MHz at the Farad-1’s 50 MHz clk — well under the W25Q64BV’s 80 MHz ceiling); done one cycle; CS owned by the controller, not here. |
| w25q64_controller | op, slot_addr[23:0], len[9:0], data FIFO side, busy, done, error[2:0] | FSM: IDLE → WREN (for writes) → COMMAND+ADDRESS → DATA or POLL_BUSY → STATUS_CHECK/TIMEOUT → DONE/ERROR. Owns CS across the full command. Busy-poll timeout 1 s of clk cycles. Page program split at 256 bytes happens above this module (loader) — this module refuses len > 256 for program and refuses any command whose len would cross a page (assertion + ERROR). |
| s8_boot_ctrl | mode_sync, uart FIFO side, spi controller side, program RAM loader port, cpu_reset_hold, boot_status | Top-level boot FSM (Section 11.2), packet parser (Section 12), image validation and copy, metadata commit. The largest control module; keep the packet parser as a clearly separated always block or submodule. |

# 10. UART subsystem

## 10.1 Baud generation

16× oversampling with a phase accumulator:

every clk:  acc <= acc + 16*BAUD
            if acc >= CLK_HZ: acc <= acc - CLK_HZ; tick16 <= 1
            else tick16 <= 0

On the Farad-1 (CLK_HZ = 50,000,000, BAUD = 115200): addend = 16 × 115200 = 1,843,200; tick16 fires on average every 27.12 clk cycles (the naive integer view — 434.028 clocks per bit — never enters the design). The long-term average is exactly BAUD, with bounded instantaneous jitter of one clk period. CLK_HZ stays a parameter so simulation can use any clock. RX and TX share one baud_gen instance only if both use the same phase; simpler and preferred: one instance, one tick16 wire, both consume it.

## 10.2 Framing

8 data bits, no parity, 1 stop bit; LSB first on the wire. RX: two-flop synchronize uart_rxd at the top level (before any edge logic); on falling edge, wait 8 ticks (half bit), confirm start still low (else return to idle — glitch rejection); sample each data bit at mid-bit (16 ticks apart); check stop bit at mid-stop; framing error flag if stop ≠ 1 (byte still delivered, flagged). TX: standard serializer; tready deasserts while the shift register is busy and the one-byte holding register is full.

## 10.3 FIFOs and errors

RX and TX FIFOs are 16 bytes deep. RX overrun: pushing into a full FIFO drops the new byte and latches the overrun sticky bit (UART_STATUS bit3). Framing errors latch bit2. Both sticky bits clear when the application reads UART_DATA. FIFO full/empty flags feed UART_STATUS bits 0/1. The bootloader depends on these semantics: it polls RX_VALID, drains promptly, and treats sticky error bits as a reason to abort the current frame (Section 12.5).

# 11. Mode selection and boot sequence

## 11.1 MODE pin semantics

MODE is a boot strap, not a run-time switch. It is sampled through a two-flop synchronizer during reset and latched when reset deasserts. Farad-1 note (Section 5, B6): there is no dedicated MODE pin — DIP0 (p84) plays the role, raw-high when open; s8_top therefore parameterizes MODE_INVERT = 1 so that the logical polarity below (1 = programming) holds: switch closed → programming, switch open → run application (board default).

- Latched MODE = 1 → CPU held in reset; boot controller enters PROG (programming) state.

- Latched MODE = 0 → boot controller enters BOOT (application start) state.

During RUN, MODE transitions are ignored until the next hard reset. The RUN packet command and CPU_CONTROL.soft_reset perform a warm hand-off: soft reset re-enters the BOOT flow using the latched mode value, so a warm reset on a board strapped to MODE=1 returns to the bootloader, and MODE=0 returns to application validation. There is no way to enter PROG without a hard reset with MODE high — this is deliberate (it prevents a running application from being knocked into programming state by a serial glitch).

## 11.2 Boot flow

POWERON / hard reset
   │
   ▼
INIT: synchronize rst_n, sample+synchronize MODE (≥2 clk settle)
   │
   ├─ latched MODE=1 ──► PROG: cpu_reset held; packet loop:
   │                        HELLO ⇄, BEGIN(erase staging slot),
   │                        DATA×N (program), VERIFY (readback CRC),
   │                        COMMIT (metadata), RUN (release CPU)
   │
   └─ latched MODE=0 ──► BOOT:
                            read JEDEC ID (0x9F) → compare family (B8)
                            read both metadata records
                            validate (magic, version, slot match, marker,
                                      length ≤ 8192, CRC16 over image)
                            pick highest generation valid record
                            copy image from slot → program RAM
                            ▲ if no valid image: set BOOT_ERR, fall back to
                              PROG (a board with no image must still accept uploads)
                            release cpu_reset → RUN

RUN state: boot controller idles; CPU_CONTROL writes are honored (resume after HALT; soft reset re-enters BOOT). UART remains owned by uart_regs; the application polls it as the echo program does.

## 11.3 Copy details

The loader reads the chosen slot with op READ (0x03) in chunks of ≤ 512 bytes (two flash pages) into program RAM words, honoring the even-byte-high rule of Section 7.1. The copy is verified by the CRC computed during the metadata validation read (recompute during copy; mismatch → BOOT_ERR=2 and fall back to the other slot, then to PROG).

# 12. Packet protocol v1 (byte-exact)

All multi-byte fields are big-endian.

## 12.1 Frame layout

| Offset | Field | Size | Definition |
| --- | --- | --- | --- |
| 0 | SYNC0 | 1 | 0x53 (‘S’) |
| 1 | SYNC1 | 1 | 0x38 (‘8’) |
| 2 | VERSION | 1 | 0x01 |
| 3 | COMMAND | 1 | See 12.3 |
| 4 | SEQ | 2 | Monotonic host counter, starts at 1 per session (per BEGIN) |
| 6 | ADDR | 4 | Command-specific (see 12.3) |
| 10 | LEN | 2 | Payload length, 0..256 |
| 12 | PAYLOAD | LEN | Command data |
| 12+LEN | CRC16 | 2 | CRC over bytes [2 .. 11+LEN] (VERSION through end of PAYLOAD) |

Maximum frame size: 12 + 256 + 2 = 270 bytes.

## 12.2 CRC16

CRC16-CCITT-FALSE: polynomial 0x1021, initial value 0xFFFF, no input reflection, no output reflection, no final XOR, MSB-first byte processing, result transmitted high byte first. Check value: CRC16(“123456789”) = 0x29B1. Both the Rust crate and the RTL CRC unit must reproduce this check value in tests before any packet work starts.

## 12.3 Commands

| CMD | Name | ADDR | LEN / PAYLOAD | Device action |
| --- | --- | --- | --- | --- |
| 0x01 | HELLO | 0 | 0 | Session handshake; ACK payload: [proto_ver, max_payload_hi, max_payload_lo, features, mfr_id, dev_id] (6 bytes; mfr/dev from the last JEDEC read). |
| 0x02 | BEGIN | slot (0=A, 1=B) | 4: image_len (2), image_crc16 (2) | Validates len (2..8192, even); erases the two sectors of the chosen slot; arms staging state. |
| 0x03 | DATA | image byte offset | 1..256: raw bytes | Offset must be a multiple of 256 and offset+len ≤ image_len. Programs one flash page. |
| 0x04 | VERIFY | 0 | 0 | Reads back the staged image, computes CRC16, compares with BEGIN’s image_crc16. ACK payload on success: [crc_hi, crc_lo]. |
| 0x05 | COMMIT | 0 | 0 | Writes the metadata record for the staged slot (Section 13.5), marker byte last. |
| 0x06 | RUN | 0 | 0 | Warm hand-off (Section 11.1): releases CPU reset if an image is committed; otherwise NAK BAD_STATE. |
| 0x07 | ABORT | 0 | 0 | Discards staging state; no flash changes after this point. |
| 0x08 | STATUS | 0 | 0 | ACK payload: [state, last_error, gen_hi, gen_mid, gen_mid2, gen_lo] (6 bytes). |

## 12.4 Responses

Every command receives exactly one response frame, using the same framing, with:

- COMMAND = request COMMAND | 0x80,

- SEQ echoed,

- ADDR echoed,

- LEN = 1 (+ command-specific data),

- PAYLOAD[0] = status code; any additional bytes follow (see the command table).

| Status | Name | Meaning |
| --- | --- | --- |
| 0x00 | OK | Command accepted and completed |
| 0x01 | BAD_CRC | Frame CRC mismatch (frame dropped silently; no ACK is sent — the host retries by timeout) |
| 0x02 | BAD_LEN | LEN invalid for the command or exceeds the negotiated maximum |
| 0x03 | BAD_SEQ | Sequence number out of order (not the duplicate case below) |
| 0x04 | BAD_OFFSET | DATA offset unaligned, out of range, or not the expected next offset |
| 0x05 | BAD_STATE | Command not valid in the current session state (e.g. DATA before BEGIN, COMMIT before VERIFY) |
| 0x06 | FLASH_ERR | SPI/flash operation failed (timeout, verify mismatch) |
| 0x07 | NO_IMAGE | RUN requested but no committed image exists |
| 0xFF | UNKNOWN | Unrecognized command or internal decode failure |

BAD_CRC frames produce no response (the device cannot trust the frame enough to attribute it); the host discovers this by ACK timeout.

## 12.5 Sequencing, duplicates, timeouts

- SEQ starts at 1 with each BEGIN and increments by 1 per host frame. The device accepts: SEQ == expected (normal) and, for DATA only, SEQ == expected − 1 with identical ADDR and LEN (duplicate retransmission) → ACK OK, payload ignored. Any other SEQ → BAD_SEQ.

- DATA offsets must arrive in order: the device tracks next_offset; a frame whose ADDR equals next_offset advances it; a frame whose ADDR equals the previous offset is the duplicate case; anything else is BAD_OFFSET.

- Device inter-byte timeout: 250 ms between bytes of one frame → the partial frame is discarded, state unchanged, parser returns to SYNC hunt. Staging state (post-BEGIN) persists across frame aborts until ABORT or COMMIT or a new BEGIN.

- Host: ACK timeout 2 s, up to 3 retransmissions of the identical frame (safe because duplicates are idempotent). Host aborts the session on the third consecutive failure.

- Sticky RX error bits (framing/overrun) observed at any point cause the device to discard the partial frame and resynchronize.

- Ordering enforced by the device: HELLO may arrive in any state; DATA/VERIFY/COMMIT require staging; RUN requires a committed image or an already-valid boot image.

# 13. W25Q64 SPI flash subsystem

## 13.1 Commands used

| Command | Opcode | Protocol |
| --- | --- | --- |
| Read JEDEC ID | 0x9F | Send 0x9F, read 3 bytes (mfr, type, capacity). Expected for the Farad-1’s W25Q64BV: 0xEF 0x40 0x17; mismatch → BOOT_ERR=3. |
| Read Status Register-1 | 0x05 | Send 0x05, read 1 byte while CS low; bit0 = WIP (busy), bit1 = WEL. |
| Write Enable | 0x06 | Single opcode; sets WEL. Required before program/erase. |
| Read Data | 0x03 | Opcode + 24-bit address + streamed read bytes (any length). |
| Page Program | 0x02 | WREN first; opcode + 24-bit address + 1..256 data bytes; bytes wrap inside the page if the host overruns — the controller must never emit more than 256 bytes or cross a page boundary (Section 9). |
| Sector Erase | 0x20 | WREN first; opcode + 24-bit sector address; then poll WIP with timeout. |

Timing bounds used by the controller (generous superset of datasheet maxima): page program ≤ 3 ms, sector erase ≤ 400 ms, WIP poll timeout 1 s overall. WP#/HOLD# (if bonded) are tied or driven per B8; the controller assumes they are inactive.

## 13.2 Byte engine vs command controller

Two modules, as in the original design intent: spi_byte_engine does bit-level work only (mode 0: MOSI changes on falling SCLK edge, MISO sampled on rising edge, SCLK idle low, MSB first, programmable divider). w25q64_controller owns CS# for the entire command duration, sequences opcodes/addresses/data, performs WREN, polls status, enforces timeouts, and reports a 3-bit error code. The byte engine must never raise CS; the controller must never shift bits.

## 13.3 Controller FSM

IDLE ──op──► [WREN] ──► SEND_OP_ADDR ──► TRANSFER (program/read)
   ▲                                          │
   │                                          ▼
   │                                    WAIT_BUSY (WIP poll, timeout)
   │                                          │
   ├── DONE ◄── READBACK_OK (verify step) ◄───┤
   └── ERROR (timeout / protection / verify) ◄┘

Erase skips TRANSFER and goes straight to WAIT_BUSY. Program issues WREN, sends op+addr+data, then WAIT_BUSY, then (for loader-initiated programs) participates in the VERIFY_READBACK step of the boot controller.

## 13.4 Storage layout (Farad-1: SHARED configuration flash — relocated base)

On the Farad-1 the W25Q64BV is the FPGA configuration flash (Section 5, B7): the bitstream sits at 0x000000 (~338 KB for the LX9 — measure the generated .bit and confirm). S8 storage therefore uses a relocated, parameterized layout. The base S8_FLASH_BASE defaults to 0x060000 (sector 192 — ≈48 KiB above a typical LX9 bitstream end) and lives in one shared constant set (flash_layout in RTL, mirrored in the Rust s8-image crate); scattered absolute addresses are forbidden.

| Range (default base) | Size | Use |
| --- | --- | --- |
| 0x000000–0x05FFFF | 384 KiB | FPGA configuration region — S8 must never erase or program |
| 0x060000–0x060FFF | 4 KiB | Metadata record sector A (record at offset 0) |
| 0x061000–0x061FFF | 4 KiB | Metadata record sector B (record at offset 0) |
| 0x062000–0x063FFF | 8 KiB | Application slot A |
| 0x064000–0x065FFF | 8 KiB | Application slot B |
| 0x066000–0x7FFFFF | rest | Reserved; untouched by v1 |

Rules: (1) before Phase 5 leaves simulation, confirm BASE ≥ (bitstream bytes rounded up to a 4 KiB sector) + 1 sector using the actual .bit file; if a future design grows the bitstream, raise BASE — never shrink it. (2) The bootloader, packet protocol (ADDR fields stay image-relative — unchanged), and host image packer all consume the base from the single constant set. (3) The iMPACT .mcs flow writes from address 0; never regenerate an .mcs with a shifted start address.

Rationale for separate metadata sectors: a commit erases only the inactive record’s sector, so the other record survives a power cut at any point — the redundancy actually works. Slots are 8 KiB = program RAM capacity exactly (fixes original issue P2).

## 13.5 Metadata record (32 bytes, at offset 0 of a metadata sector)

| Offset | Size | Field |
| --- | --- | --- |
| 0 | 4 | Magic “S8MT” = 53 38 4D 54 |
| 4 | 1 | Format version = 0x01 |
| 5 | 1 | Slot id (0 = A, 1 = B) |
| 6 | 4 | Generation counter, u32 big-endian, monotonically increasing |
| 10 | 2 | Image length in bytes, u16 BE, 2..8192, even |
| 12 | 2 | Image CRC16 (CCITT-FALSE over the image bytes) |
| 14 | 17 | Reserved, must read 0xFF |
| 31 | 1 | Validity marker = 0xA5, programmed last |

A record is valid iff: magic matches, version matches, slot id matches its sector’s associated slot, marker == 0xA5, length in range, and the slot image’s CRC16 equals the record’s CRC. Invalid ⇒ treat the sector as empty (0xFF or garbage are both rejected by the magic check first).

## 13.6 Atomic update procedure (device side, on COMMIT)

- Read back the full staged image; recompute CRC16; compare to the staged CRC from BEGIN. Mismatch → FLASH_ERR, staging kept (host may retry VERIFY/ABORT).

- Choose the target record sector: the one whose record is not the current newest valid record (alternates naturally by generation parity).

- Erase the target record sector (0x20 + timeout).

- Program record bytes 0..30 (all but the marker); read back and compare.

- Program the marker byte (offset 31); read back and confirm 0xA5.

- Only now is the new image “committed”. A power cut before step 5 leaves the old record intact and the new one invalid — boot falls back to the previous image (verified by the Phase-6 power-cut test).

The currently active slot is never erased: BEGIN for slot X requires that slot X is not the one the device booted from (NAK BAD_STATE otherwise). Alternating A/B across updates guarantees the previous image is still present when the new one is committed.

## 13.7 Boot-time validation (recap of the algorithm)

Read both records; filter to valid ones; pick the highest generation; verify its image CRC during (or after) the copy to program RAM; on copy-time mismatch, fall back to the other valid record; if none, BOOT_ERR=1 and fall back to PROG. Generation comparison is unsigned u32; wrap-around is accepted as a non-issue (centuries at human update rates).

# 14. Rust host toolchain

## 14.1 Workspace

host/s8/ is a Cargo workspace: s8-isa, s8-assembler, s8-image, s8-serial, s8-disasm, s8-cli. Dependencies kept minimal: clap (CLI), serialport (host serial), thiserror (errors), and an in-repo CRC16 implementation in s8-image (do not pull a generic crc crate — the parameters are part of the protocol spec and must be visible in-tree, pinned by the 0x29B1 test).

## 14.2 s8-isa

The single source of truth for encodings.

pub enum Instruction { Ldi { rd: u8, imm: u8 }, Add { rd: u8, rs: u8 }, /* ...all 20-ish variants... */ }
pub fn encode(i: &Instruction) -> u16;
pub fn decode(w: u16) -> Result<Instruction, DecodeError>; // errors on any illegal pattern per 6.2
pub fn encode_str(s: &str) -> Result<u16, AsmError>;       // test helper: "LDI R0, 1" -> 0x1001

Tests: the full golden-vector table of Section 6.7 (generated file included via include_str!), plus encode→decode→encode round trips over all legal encodings (exhaustive over the legal subspace), plus rejection tests for every illegal pattern class.

## 14.3 s8-assembler

Two-pass, per Section 6.8. Pass 1 assigns word addresses and collects labels/.equ constants; pass 2 parses operands, validates ranges, applies relaxation, encodes via s8-isa. Output: flat binary (big-endian word packing), optional Intel HEX, and an optional listing file (address, word, source line). Mandatory diagnostics with file:line:col. Tests: forward references, duplicate/undefined labels, range violations for each immediate format, branch relaxation both directions, .org overlap, .byte packing order, image-size limit (8192), and every example program in Section 6.9 assembling to its normative word list.

## 14.4 s8-image

.s8img container, 12-byte header + payload:

| Offset | Size | Field |
| --- | --- | --- |
| 0 | 4 | Magic “S8IM” = 53 38 49 4D |
| 4 | 1 | Format version = 0x01 |
| 5 | 1 | Flags (bit0: payload is a raw binary; others reserved 0) |
| 6 | 1 | Preferred slot (0=A, 1=B, 0xFF=unspecified) |
| 7 | 1 | Reserved = 0 |
| 8 | 2 | Payload length, u16 BE (2..8192, even) |
| 10 | 2 | Payload CRC16 (CCITT-FALSE) |
| 12 | N | Payload (program bytes, even-byte-high word packing) |

pack(bin, slot) -> S8img / unpack(s8img) -> (bin, slot) with full validation. Tests: pack/unpack round trip, CRC corruption detection, length-limit rejection.

## 14.5 s8-serial

pub trait Transport { fn write(&mut self, buf: &[u8]) -> io::Result<()>; fn read_timeout(&mut self, buf: &mut [u8], ms: u32) -> io::Result<usize>; }
pub struct PortTransport { /* wraps serialport::SerialPort */ }
pub struct MockTransport { /* scripted request/response pairs for tests */ }
pub struct Client<T: Transport> { /* packet codec + session FSM */ }
impl<T: Transport> Client<T> {
    pub fn hello(&mut self) -> Result<Hello, ProtoError>;
    pub fn upload(&mut self, img: &S8img, slot: Slot, progress: impl FnMut(usize, usize)) -> Result<(), ProtoError>;
    pub fn verify(&mut self) -> Result<u16, ProtoError>;
    pub fn commit(&mut self) -> Result<(), ProtoError>;
    pub fn run(&mut self) -> Result<(), ProtoError>;
    pub fn status(&mut self) -> Result<Status, ProtoError>;
}

Implements Section 12 exactly: CRC, big-endian fields, SEQ management, duplicate-idempotent DATA retries (3 × 2 s), BAD_* handling with distinct user-facing errors. The serial crate is isolated behind Transport so the protocol logic is fully unit-tested with MockTransport — no hardware, no timing flakiness. Tests: golden frame bytes for HELLO/BEGIN/DATA(1 page)/VERIFY/COMMIT/RUN (hand-computed vectors included in the spec repo), timeout/retry paths, BAD_CRC silent-drop path, duplicate-DATA idempotency, BAD_OFFSET on a skipped page.

## 14.6 s8-disasm

decode-driven; annotates labels when given the symbol map; used by s8 disasm and in round-trip tests (assemble → disassemble → reassemble → identical bytes) over all example programs.

## 14.7 s8-cli

s8 asm examples/gpio.s8 -o build/gpio.bin [--listing build/gpio.lst] [--hex]
s8 disasm build/gpio.bin
s8 image build/gpio.bin --slot A -o build/gpio.s8img
s8 upload build/gpio.s8img --port COM5 --baud 115200      # port auto-scan allowed if --port omitted
s8 verify --port COM5
s8 status --port COM5
s8 monitor --port COM5 --baud 115200                      # raw terminal, local echo off, hex dump option

Port names: never hard-code COM5 or /dev/ttyUSB0; enumerate candidates with serialport::available_ports() and require confirmation when ambiguous. cargo test must pass with no hardware attached (all serial tests use MockTransport).

# 15. Xilinx ISE workflow

ISE 14.7 is the final release supporting Spartan-6; Vivado does not. Practical notes: the Windows installer ships both 32-bit and 64-bit variants (use the 32-bit toolchain on modern Windows if the 64-bit GUI misbehaves, or run in a Windows 7/10 VM or under Linux Wine); iSIM and the command-line flows (xst, ngdbuild, map, par, trce, bitgen) are scriptable and preferred for CI-like reproducibility over the GUI.

Sequence:

- Create project with the exact full part string (B1) — including speed grade and package.

- Add RTL to the synthesis source set; testbenches only to the simulation source set.

- Set s8_top as synthesis top; run behavioral simulation of every testbench in Section 16.1 before synthesis.

- Synthesis: verify inferred BRAM for s8_program_ram (must be BRAM, not distributed) and check utilization reports against the budget (Section 16.5).

- Constraints: start from the Farad-1 UCF below — every LOC verified against the schematic renders (Section 5.2). The vendor example UCF (BoardFiles/Farad1_UCF.ucf) is correct on its LOCs (including Serial_Tx = p41 / Serial_Rx = p40) but incomplete: it has no IOSTANDARD anywhere, no PERIOD constraint, and none of the flash pins. All three gaps are fixed here:

# S8 v1 constraints — Farad-1 (xc6slx9-2tqg144), verified against schematic rev 2016-12-29
NET "clk"        LOC = "P85" | IOSTANDARD = LVCMOS33;  TNM_NET = sys_clk;
TIMESPEC "TS_sys_clk" = PERIOD "sys_clk" 20.000 ns HIGH 50%;   # 50.000 MHz oscillator
NET "rst_n"      LOC = "P87" | IOSTANDARD = LVCMOS33;  # nFP_RST button, active low, RC-debounced
NET "mode"       LOC = "P84" | IOSTANDARD = LVCMOS33;  # DIP0; open=1, closed=0; MODE_INVERT=1 in s8_top
NET "gpio<0>"    LOC = "P105" | IOSTANDARD = LVCMOS33; # LED0, active high
NET "gpio<1>"    LOC = "P104" | IOSTANDARD = LVCMOS33; # LED1, active high
NET "gpio<2>"    LOC = "P102" | IOSTANDARD = LVCMOS33; # LED2, active high
NET "gpio<3>"    LOC = "P101" | IOSTANDARD = LVCMOS33; # LED3, active high
NET "uart_rxd"   LOC = "P83" | IOSTANDARD = LVCMOS33;  # FT2232H Ch.B TXD (after FT_PROG -> UART)
NET "uart_txd"   LOC = "P82" | IOSTANDARD = LVCMOS33;  # FT2232H Ch.B RXD
# DB9/MAX232 fallback: uart_rxd = P40, uart_txd = P41 (swap LOCs only)
NET "spi_sck"    LOC = "P70" | IOSTANDARD = LVCMOS33;  # flash CK (CCLK pin, net FP_PR_CLK)
NET "spi_cs_n"   LOC = "P38" | IOSTANDARD = LVCMOS33;  # flash CS# (CSO_B; board pull-up R6)
NET "spi_mosi"   LOC = "P64" | IOSTANDARD = LVCMOS33;  # flash DI (net MISO0)
NET "spi_miso"   LOC = "P65" | IOSTANDARD = LVCMOS33;  # flash DO (net MISO1)
NET "spi_wp_n"   LOC = "P62" | IOSTANDARD = LVCMOS33;  # flash WP# - drive high, no board pull-up
NET "spi_hold_n" LOC = "P61" | IOSTANDARD = LVCMOS33;  # flash HOLD# - drive high, no board pull-up

- Implementation + timing: all paths must meet the PERIOD constraint with margin; investigate every timing summary line before generating a bitstream.

- Bitstream via iMPACT/JTAG; keep a known-good recovery bitstream in build/ with its hash recorded.

- Reproducibility record per bitstream: ISE version, git revision, UCF content, timing summary, utilization summary, bitstream SHA-256.

UG380 (Spartan-6 Configuration User Guide) is required reading: the Farad-1’s W25Q64BV is the configuration flash (Section 5, B7) and S8 accesses it as ordinary user I/O after configuration completes — CCLK/CSO_B/MISO0-style pins become fabric-driven SPI signals per the UCF above.

# 16. Verification plan

## 16.1 RTL testbenches

| Testbench | Covers | Key checks |
| --- | --- | --- |
| tb_s8_alu | s8_alu | Every operation class against a behavioral model; exhaustive operand sweep for ADD/SUB (all 65536 pairs) verifying flags per Section 6.5 formulas; directed cases for each O/C corner (0x7F+1, 0x80−1, 0xFF+1, equal operands, etc.). |
| tb_s8_regfile | s8_regfile | Write/read ports, sync-write timing, no read-during-write hazard. |
| tb_s8_cpu_programs | CPU + program RAM | Loads normative programs from tb/vectors/ (generated from s8-isa): GPIO words execute and drive the port value; UART echo program run against a UART BFM that injects bytes and checks echoes; golden vector sweep: one instruction per encoding, architectural state checked after each; HALT/resume; TRAP on opcode 0xE and on non-zero reserved bits; stack fault TRAP. |
| tb_uart_loopback | rx + tx + regs + baud_gen | Tie txd→rxd; stream 256 random bytes with random gaps; check data integrity, FIFO full behavior, sticky overrun/framing bits, framing error on a corrupted stop bit, tolerance to ±2% injected baud offset. |
| tb_spi_byte_engine | spi_byte_engine | Mode-0 timing against a bit-accurate reference; MOSI/MISO ordering; divider correctness for DIV ∈ {2,4,8}. |
| tb_w25q64_model | behavioral flash model | The model itself: JEDEC ID read, WEL/WIP behavior, page wrap protection, sector erase, busy timing, and its own fault-injection hooks. |
| tb_boot_packets | packet parser + CRC + session FSM | Golden CRC (0x29B1 check vector first); golden frame bytes from Section 14.5’s test vectors; fuzz/negative: bad sync, bad CRC, bad LEN, bad SEQ, duplicate DATA (idempotent), skipped page (BAD_OFFSET), commands out of order (BAD_STATE), truncated frames (timeout recovery), oversize LEN. |
| tb_full_boot | entire boot path | MODE=1: full upload session against tb_w25q64_model, COMMIT, RUN, then MODE=0 cold boot: image copied to program RAM, CPU executes it and emits the expected UART bytes; power-cut injection at each COMMIT step → previous image still boots; slot-selection: newer generation wins; corrupted image → fallback. |

## 16.2 Behavioral W25Q64 model (tb_w25q64_model.v)

A Verilog model with an 8 MiB array (or 64 KiB sparse window — the layout only needs 0x0000–0x5FFF plus wrap tests — choose sparse to keep sim fast, but keep addresses honest). Behaviors: JEDEC ID bytes (parameterized, default a Winbond ID), WEL/WIP status semantics, page-program bit-clear-only semantics (programming over non-errored bits must be modeled as the real device behaves: AND semantics), sector erase timing (parameterized fast for sim), busy delay parameters, and fault hooks (drop N CS edges, delay busy N cycles, corrupt byte at address X) driven by plusargs.

## 16.3 Host-side tests

All cargo test suites run offline: golden vectors, assembler diagnostics, image pack/unpack, protocol sessions over MockTransport, retry/timeout logic with simulated clock via injectable sleep. A --locked build must be reproducible.

## 16.4 Hardware acceptance tests (per phase)

| Phase | Test | Pass |
| --- | --- | --- |
| 2 | GPIO program on board | LED0 (p105) lights and holds the programmed level: LDI R0,1 + OUT GPIO_OUT,R0 turns it on (active high); scope confirms static level after the loop |
| 3 | UART loopback | Board echoes PC bytes at 115200 for ≥ 10 min with zero losses at the offered rate |
| 3 | Framing stress | Host sends 0x00, 0xFF, 0x55, 0xAA patterns; echo integrity maintained |
| 5 | JEDEC ID | s8 status (or UART console command in the boot controller) reports the expected family ID |
| 6 | Full upload | s8 upload + s8 verify + power-cycle with MODE=0 → application runs from flash |
| 6 | Power-cut sweep | Cut power during COMMIT at ≥ 5 randomized delays; board always boots either the old or the new image, never a bricked state |
| 6 | Slot alternation | Two successive uploads land in A then B; generation increments monotonically |

## 16.5 Budgets and traceability

Budgets (XC6SLX9: 1,430 slices, 32 BRAM18 blocks, 90 Kb distributed RAM — Section 5.4): program RAM = 4 BRAM18; UART FIFOs + data RAM = distributed RAM (0 BRAM); total slice utilization target ≤ 60 % (~860 slices) at the end of Phase 6 (margin for the compiler/debugger phases).

Every numbered requirement in this document maps to at least one testbench or acceptance test; the mapping table is maintained in docs/verification-plan.md with columns (requirement tag, section, test ID, status). Requirement tags are assigned per section (e.g. S8-12.5-03 = Section 12.5, third bullet).

# 17. Safety, reliability, and failure handling

Consolidated hard rules (each is testable; several are already in the phase gates):

- Pin assignments come from the schematic only; every top-level port gets LOC + IOSTANDARD; no placeholder survives into implementation.

- 3.3 V logic levels only on user I/O; never RS-232 voltages; never 5 V unless the bank spec explicitly allows it.

- CPU and bootloader ownership are mutually exclusive: CPU reset is held for the entire PROG state.

- Every length, address, loop, and wait is bounded; the assembler rejects images > 8192 bytes; the device rejects LEN > 256 and offsets ≥ image_len.

- The active slot is never erased; the metadata marker is written last; both metadata records live in separate erase sectors.

- Flash BUSY is always polled with a timeout; WEL is checked; programmed data is read back.

- The FPGA configuration region is untouchable — the flash IS shared on the Farad-1 (B7); layout constants live in one place (S8_FLASH_BASE, Section 13.4).

- JTAG reconfiguration with a known-good recovery bitstream is always possible (never lock configuration pins as user I/O in the constraints).

- All asynchronous inputs pass two-flop synchronizers (MODE, rst_n, uart_rxd, miso sampled by the divider — synchronous to sck by construction).

- CRC detects corruption; it is not authentication. Document this wherever uploads are described.

- Farad-1 flash pins WP# (p62) and HOLD# (p61) have no board pull-ups: the boot controller drives both HIGH from configuration release onward, before any flash command. A floating HOLD# can stall the SPI bus.

- The DB9/MAX232 path is electrically safe as designed (5 V translator, but the FPGA-side RX passes through a 5.6 K/10 K divider ≈ 3.2 V max, input-only). Never bypass the divider; never treat the DB9 pins as logic-level.

# 18. Phased execution plan

Each phase has a deliverable and a hard exit gate. Do not start the next phase until the gate is demonstrated, ideally by an automated test.

| Phase | Deliverable | Exit gate |
| --- | --- | --- |
| 0. Foundation | This spec frozen in docs/; board facts B1–B9 verified or explicitly deferred; vector generator running | Golden vectors reproduce in both Node generator and Rust tests; UCF skeleton exists with placeholders still unfilled |
| 1. CPU core | s8_alu, s8_regfile, s8_cpu, s8_program_ram, s8_data_ram, s8_bus + testbenches | tb_s8_alu, tb_s8_regfile, tb_s8_cpu_programs pass; golden vector sweep green; HALT/TRAP/stack-fault cases green |
| 2. First hardware | s8_top minimal (CPU + GPIO), Farad-1 UCF from Section 15, ISE project runs clean | GPIO demo (Section 6.9.1) observed on LED0 (p105, active high); timing closure with margin; utilization recorded |
| 3. UART | baud_gen, uart_rx, uart_tx, uart_regs, echo program | tb_uart_loopback green; on-board echo survives the 10-minute stress test |
| 4. Boot protocol | s8_boot_ctrl packet parser + session FSM; host s8-serial + s8-cli upload/verify commands | tb_boot_packets green (including fuzz/negative); a full host↔RTL upload session runs against the flash model in simulation |
| 5. SPI discovery | spi_byte_engine, w25q64_controller, JEDEC read path wired into the boot controller | tb_spi_byte_engine and model tests green; on-board JEDEC read returns 0xEF 0x40 0x17 (if not: continuity check + p64/p65 swap per Section 5.3) |
| 6. Persistent slots | Full loader, metadata commit, A/B selection, s8 image/s8 upload end-to-end | tb_full_boot green incl. power-cut injection; hardware acceptance table (16.4) passes; two alternating uploads verified on hardware |
| 7. Compiler | C-like frontend per Section 19 | Compiled programs match assembler-level equivalents in simulation; ABI conformance tests pass |
| 8. Debugger | s8 disasm symbols, monitor improvements, status/break support | Repeatable debug workflow demonstrated: upload → run → observe → diagnose from symbols |

Phase ordering rationale: simulation-heavy phases (1, 4) proceed without board facts; hardware risk is retired early (Phase 2 proves the toolchain, constraints, and board interface before any protocol work); flash risk lands late, once the recovery path (JTAG + known-good bitstream) is already proven.

# 19. Future work (specified, not scheduled)

## 19.1 Compiler (Phase 7)

Pipeline: source → lexer → parser/AST → semantic analysis (u8, bool, void; no dynamic allocation) → IR → optional constant folding / DCE → instruction selection (S8 ISA v1) → register allocation → branch relaxation → s8-isa encoding → .s8img → existing uploader. Statements: declarations, assignment, if/else, while, functions, return. Built-ins only when backed by real runtime/hardware: gpio_read, gpio_write, uart_available, uart_read, uart_write, delay. Debug info: JSON symbol map (source line ↔ word address) feeding s8 disasm.

## 19.2 Instruction-fetch from external flash

If images must exceed 8 KiB: an XIP fetch interface over the SPI controller with a simple direct-mapped instruction cache is the specified direction. Explicitly deferred; BRAM copy remains the v1 mechanism.

## 19.3 ABI proposal (freeze when the compiler starts)

R0 = return value and scratch; R1–R2 = arguments; R3–R5 = caller-saved temporaries; R6–R7 = callee-saved; SP grows down from 0xEF; 16-entry hardware stack implies a software spill strategy for deep call chains (compiler must either prove depth ≤ 16 or spill to 0x00–0xDF); prologue = PUSH callee-saved, epilogue = POP in reverse.

# 20. References

- UG380 — Spartan-6 FPGA Configuration User Guide. Configuration modes, SPI flash wiring, user-mode access rules. Required: the Farad-1’s W25Q64BV IS the configuration flash (B7); S8 accesses it as user I/O after configuration. https://docs.amd.com/v/u/en-US/ug380

- UG382 — Spartan-6 FPGA Memory Resources User Guide. BRAM primitives, port configurations, inference rules for s8_program_ram/s8_data_ram.

- UG612 — ISE Synthesis and Simulation Design Guide; UG621 — ISim simulation guide (command-line flows preferred).

- Xilinx ISE Design Suite 14.7 (final Spartan-6 release): https://www.xilinx.com/products/design-tools/ise-design-suite.html

- Spartan-6 package pinout files: https://www.xilinx.com/support/package-pinout-files/spartan-6-pkgs.html (board routing and bank voltages still require the schematic).

- Winbond W25Q64 family datasheet (use the exact suffix): https://www.winbond.com/hq/product/code-storage-flash-memory/serial-nor-flash/?__locale=en

- CRC16-CCITT-FALSE parameters and check value 0x29B1 (canonical catalogue value; reproduced in-tree by s8-image’s CRC unit test).

- Farad-1 board documentation (in-repo): BoardFiles/Farad-1-Schematic.pdf (6 sheets, rev 2016-12-29), BoardFiles/Farad-1-User-Manual.pdf, BoardFiles/Farad1_UCF.ucf (vendor example: LOCs correct, missing IOSTANDARD/PERIOD constraints — build from Section 15 instead).

# Appendix A — Guide for an implementing agent

- Start at Phase 0/1: create the repo tree (Section 3), copy the frozen tables (Sections 6, 12, 13.5) into docs/, and run tools/gen_isa_vectors.js to populate tb/vectors/ and the Rust golden-vector include file.

- Build s8-isa first and make its tests pass; everything else depends on it.

- Build s8_alu + tb_s8_alu next; the exhaustive flag sweep catches formula misunderstandings before they spread into the CPU.

- When writing the CPU FSM, keep the state list of Section 8.2 verbatim; add states only with a written justification in the module header.

- The two most bug-prone seams in the whole design are (a) the byte order between image bytes and 16-bit words (even byte = high byte — one helper function, used everywhere, no ad-hoc swapping) and (b) the DATA packet offset/page-boundary rules (256-aligned offsets, device rejects everything else). Test both seams directly, not just through the happy path.

- Never “fix” a failing golden vector by editing the vector. The vector is the spec; the code is suspect.

- Board facts for the Farad-1 are already verified and recorded in Section 5 (evidence: schematic page renders inspected visually, page by page). On first bring-up, perform the Section 5.3 checks — FT_PROG EEPROM backup then Channel B → UART; flash-pin JEDEC confirmation; record the results in docs/board-bringup.md. The vendor UCF ships without IOSTANDARD/PERIOD constraints: always build from the Section 15 UCF.

End of specification.